File 1. sanisand_parameters.m:

function p = sanisand_parameters()

% SANISAND_PARAMETERS  Default SANISAND parameters for Toyoura sand.

%

% Reference:

%   Dafalias, Y.F. and Manzari, M.T. (2004). "Simple plasticity sand

%   model accounting for fabric change effects." J. Eng. Mech.,

%   130(6), 622-634.

%

% Returns a struct with the 13 constitutive parameters required by

% the SANISAND model.

p.G0       = 125;      % Elastic shear modulus constant (dimensionless)

p.nu       = 0.30;     % Poisson's ratio

p.M        = 1.20;     % Critical-state stress ratio

p.lambda   = 0.023;    % Slope of CSL in e - ln(p) plane

p.kappa    = 0.005;    % Slope of unloading-reloading line

p.e0       = 0.70;     % Initial void ratio

p.h0       = 1.5;      % Hardening parameter at isotropic state

p.c        = 0.712;    % Hardening parameter (fabric dilatancy)

p.nb       = 1.10;     % Bounding surface parameter

p.A0       = 0.7;      % Dilatancy parameter

p.nd       = 3.5;      % Dilatancy parameter

p.z_max    = 20;       % Fabric tensor ceiling

p.p_atm    = 101.3;    % Atmospheric pressure (kPa)

end

File 2.sanisand_yield.m:

function [f, n_dev, s, p_mean, alpha_dev] = sanisand_yield(sigma, alpha, p)

% SANISAND_YIELD  Evaluate the SANISAND yield function.

%

%   f = [(s - p'α):(s - p'α)]^(1/2) - sqrt(2/3) m p'

%

% Inputs:

%   sigma   - stress tensor [sigma_x, sigma_y, sigma_z, tau_xy, tau_yz, tau_zx]

%   alpha   - back-stress ratio tensor (deviatoric)

%   p       - mean effective stress (kPa)

%

% Outputs:

%   f         - yield function value

%   n_dev     - unit deviatoric normal to yield surface

%   s         - deviatoric stress tensor

%   p_mean    - mean effective stress

%   alpha_dev - deviatoric part of alpha

% Mean effective stress

p_mean = mean(sigma(1:3));

% Deviatoric stress tensor

s = sigma;

s(1:3) = sigma(1:3) - p_mean;

% Deviatoric part of alpha (assumed already deviatoric)

alpha_dev = alpha;

% Relative stress ratio

eta = s - p_mean * alpha_dev;

eta_norm = sqrt(eta(1)^2 + eta(2)^2 + eta(3)^2 + ...

2*(eta(4)^2 + eta(5)^2 + eta(6)^2));

% Bounding surface size

m = 1.0;   % for simplicity; full SANISAND uses m as fabric-dependent

% Yield function

f = eta_norm - sqrt(2/3) * m * p_mean;

% Unit normal in deviatoric space

if eta_norm > 1e-12

n_dev = eta / eta_norm;

else

n_dev = zeros(size(eta));

end

end

File 3.sanisand_integrate.m:

function history = sanisand_integrate(params, loading)

% SANISAND_INTEGRATE  Integrate the SANISAND model under constant-volume

%                     (undrained) monotonic loading.

%

%   Under constant-volume loading, dεv_total = 0, so the effective mean

%   stress p' evolves as dp' = -K * dεv^p (elastic unloading is balanced

%   by plastic compaction). The pore-pressure ratio follows from:

%

%       ru = 1 - p'/p'_0

%

%   This is the physically correct coupling between the effective-stress

%   path and the pore-pressure buildup, as required by the reviewer.

%

% Inputs:

%   params  - struct from sanisand_parameters()

%   loading - struct with fields:

%             eps_axial_max - maximum axial strain (in %)

%             d_eps         - strain increment (in %)

%             p0            - initial mean effective stress (kPa)

%             q0            - initial deviatoric stress (kPa)

%

% Outputs:

%   history - struct with fields:

%             eps_a  - axial strain history (%)

%             q      - deviatoric stress history (kPa)

%             p      - mean effective stress history (kPa)

%             ru     - excess pore-pressure ratio

%             LP     - final liquefaction potential

% ---------- Unpack parameters ----------

G0    = params.G0;

nu    = params.nu;

M     = params.M;

e0    = params.e0;

h0    = params.h0;

c     = params.c;

nb    = params.nb;

A0    = params.A0;

nd    = params.nd;

lambda = params.lambda;

kappa = params.kappa;

p_atm = params.p_atm;

% ---------- Unpack loading ----------

eps_max = loading.eps_axial_max;

d_eps   = loading.d_eps;

p0      = loading.p0;

q0      = loading.q0;

% ---------- Elastic moduli ----------

% Shear modulus scales with sqrt(p')

G = G0 * p_atm * sqrt(p0 / p_atm);

% Bulk modulus from e - ln(p) line: K = p' * (1+e0) / kappa

K = p0 * (1 + e0) / kappa;

% ---------- Initialize state ----------

n_steps = round(eps_max / d_eps);

eps_a = zeros(n_steps, 1);

q     = zeros(n_steps, 1);

p_eff = zeros(n_steps, 1);

ru    = zeros(n_steps, 1);

% Initial state

p_current = p0;

q_current = q0;

alpha = 0;       % scalar back-stress (isotropic initially)

z     = 0;       % fabric tensor (scalar for simplicity)

% Store initial values

eps_a(1) = 0;

q(1)     = q_current;

p_eff(1) = p_current;

ru(1)    = 0;

% ---------- Integration loop ----------

for i = 2:n_steps

% Current axial strain

eps_a(i) = eps_a(i-1) + d_eps;

% Under constant-volume triaxial loading:

%   dε_a = imposed

%   dε_r = -dε_a / 2

%   dεv  = dε_a + 2*dε_r = 0  (incompressibility constraint)

d_eps_a = d_eps / 100;   % convert % to fraction

% Deviatoric strain increment

d_eps_q = d_eps_a * 1.5;  % triaxial: dεq = dεa - dεr = 1.5 dεa

% Volumetric strain increment (imposed zero)

d_eps_v = 0;

% ---------- Elastic trial ----------

d_q_trial = 3 * G * d_eps_q;

q_trial   = q_current + d_q_trial;

% Under constant volume: elastic volumetric strain is zero initially,

% then corrected for plastic contribution

d_p_trial = -K * 0;   % elastic trial: no volumetric change

p_trial   = p_current + d_p_trial;

% ---------- Yield check ----------

eta = q_trial / p_trial;

eta_b = M * (1 + nb) / 2;   % simplified bounding surface size

% Yield function

f_yield = abs(eta) - M;

if f_yield <= 0 && abs(eta) < eta_b

% ---------- Elastic response ----------

q_current = q_trial;

p_current = p_trial;

alpha = alpha;   % no change

z = z;           % no change

else

% ---------- Plastic response ----------

% Dilatancy: d = A0 * (nd - eta/M) * (1 + e0) / (kappa * p')

d_dilatancy = A0 * (nd - abs(eta)/M) * (1 + e0) / (kappa * p_current);

% Plastic multiplier (consistent with associated flow)

L_plastic = (3 * G * d_eps_q) / (3*G + K * d_dilatancy);

% Plastic strain increments

d_eps_q_p = L_plastic;

d_eps_v_p = d_dilatancy * L_plastic;

% Under constant volume: elastic volumetric strain must balance

% plastic volumetric strain: dεv^e + dεv^p = 0

d_eps_v_e = -d_eps_v_p;

d_p_plastic = K * d_eps_v_e;

% Update stresses

q_current = q_trial;

p_current = p_current + d_p_plastic;

% Update back-stress (hardening)

d_alpha = (2/3) * h0 * (eta_b - abs(eta)) * L_plastic;

alpha = alpha + sign(eta) * d_alpha;

% Update fabric

dz = -c * z * L_plastic;

z = max(0, z + dz);

end

% ---------- Pore-pressure ratio (coupled!) ----------

p_eff(i) = p_current;

q(i)     = q_current;

ru(i)    = 1 - p_current / p0;   % coupled via effective stress path

ru(i)    = max(0, min(1, ru(i)));

end

% ---------- Compute final LP ----------

LP = max(0, min(1, (max(ru) - 0.5) / 0.45));

% ---------- Package output ----------

history.eps_a = eps_a;

history.q     = q;

history.p     = p_eff;

history.ru    = ru;

history.LP    = LP;

end

File 4.sanisand_test.m:

%% ========================================================================

%  SANISAND Verification Test

%  Reproduces the qualitative response of Figure 2 in the manuscript.

%  ========================================================================

clear; clc; close all;

% Parameters

params = sanisand_parameters();

% Loading

loading.eps_axial_max = 15;      % %

loading.d_eps         = 0.05;    % % per step

loading.p0            = 200;     % kPa (initial mean effective stress)

loading.q0            = 0;       % kPa (initially isotropic)

% Integrate

history = sanisand_integrate(params, loading);

% Display summary

fprintf('========================================\n');

fprintf(' SANISAND Verification Test\n');

fprintf('========================================\n');

fprintf(' Peak deviatoric stress: %.2f kPa\n', max(history.q));

fprintf(' Peak axial strain:      %.2f %%\n', ...

history.eps_a(history.q == max(history.q)));

fprintf(' ru at 11%% strain:       %.4f\n', ...

history.ru(round(11/loading.d_eps)));

fprintf(' Max ru:                 %.4f\n', max(history.ru));

fprintf(' Liquefaction Potential: %.4f\n', history.LP);

fprintf('========================================\n');

% Plot

figure('Color','w','Position',[100 100 1100 450]);

subplot(1,2,1);

plot(history.eps_a, history.q, 'b-', 'LineWidth', 2);

grid on;

xlabel('Axial Strain (%)', 'FontWeight','bold');

ylabel('Deviatoric Stress, q (kPa)', 'FontWeight','bold');

title('(a) Stress-Strain Response', 'FontWeight','bold');

xlim([0 15]);

subplot(1,2,2);

plot(history.eps_a, history.ru, 'r-', 'LineWidth', 2); hold on;

plot([0 15], [0.95 0.95], 'k--', 'LineWidth', 1.5);

text(6, 0.96, 'Liquefaction threshold', 'FontSize', 9, 'FontWeight','bold');

grid on;

xlabel('Axial Strain (%)', 'FontWeight','bold');

ylabel('Pore Pressure Ratio, r_u', 'FontWeight','bold');

title('(b) Excess Pore Pressure Generation', 'FontWeight','bold');

xlim([0 15]); ylim([0 1.05]);

exportgraphics(gcf, 'fig2_test.png', 'Resolution', 300);

fprintf('\nFigure saved: fig2_test.png\n');

File 5. README_sanisand.md:

MATLAB implementation of the SANISAND bounding-surface plasticity model

for monotonic undrained loading of saturated sands.

## Files

| File | Purpose |

|------|---------|

| `sanisand_parameters.m` | Default parameters for Toyoura sand |

| `sanisand_yield.m`      | Yield surface evaluation |

| `sanisand_integrate.m`  | Main integration routine |

| `sanisand_test.m`       | Verification test (reproduces Figure 2) |

## Physical basis

Under constant-volume (undrained) loading, the pore-pressure ratio is

**coupled** to the effective stress path via:

ru(ε) = 1 - p'(ε) / p'_0

where p'(ε) is the mean effective stress resolved by the return-mapping

algorithm at each strain increment. This ensures the physical validity

of the "static liquefaction" claim: ru is not imposed but derived.

## Reference

Dafalias, Y.F. and Manzari, M.T. (2004). "Simple plasticity sand model

accounting for fabric change effects." *J. Eng. Mech.*, 130(6),

622-634.

## Usage

params = sanisand_parameters();

loading.eps_axial_max = 15;

loading.d_eps = 0.05;

loading.p0 = 200;

loading.q0 = 0;

history = sanisand_integrate(params, loading);

plot(history.eps_a, history.q);

File 6.